SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | J. BRAHMAIAH | Reg.No.: | 192472286 |
| Department: | CSE -AI | Date: | 04-08-2026 |

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

Experiment 1: Register Transfer Operations and ALU Operations

Aim

To design and simulate a system that performs register transfer operations along with arithmetic and logical operations using an ALU.

Objective

- Study Register Transfer Language (RTL).

- Perform register-to-register data transfer.

- Execute ALU operations such as Addition, Subtraction, AND, and OR.

- Observe the role of the ALU in processing data.

Requirements

- Digital simulator (Logisim, Proteus, Multisim, or Digital Logic Simulator)

- Registers (R0, R1, R2, R3)

- Arithmetic Logic Unit (ALU)

Theory

Registers temporarily store binary data inside the CPU. Register Transfer Language (RTL) specifies the movement of data between registers.

Examples:

- R1 ← R2

- R3 ← R1 + R2

- R2 ← R1 AND R3

The ALU performs arithmetic and logical operations under the control of the control unit.
code:

R1 = 25

R2 = 10

R3 = 0

print("Initial Values")

print("R1 =", R1)

print("R2 =", R2)

R3 = R1

print("\nRegister Transfer")

print("R3 =", R3)

print("\nAddition =", R1 + R2)

print("Subtraction =", R1 - R2)

print("AND =", R1 & R2)

print("OR =", R1 | R2)

Procedure

- Create four 8-bit registers.

- Connect all registers to the ALU.

- Load sample values:

  - R1 = 25

  - R2 = 10

- Execute operations:

  - Transfer R1 → R3

  - Add R1 + R2

  - Subtract R1 − R2

  - Perform AND

  - Perform OR

- Observe register contents after each operation.

Observation Table

| Operation | Result |
| --- | --- |
| R3 ← R1 | 25 |
| R3 ← R1 + R2 | 35 |
| R3 ← R1 − R2 | 15 |
| R3 ← R1 AND R2 | 8 |
| R3 ← R1 OR R2 | 27 |

Result

The ALU successfully performed arithmetic and logical operations while registers transferred data correctly.

Experiment 2: Single Bus vs Multi Bus Organization

Aim

To compare single-bus and multi-bus computer organizations.

Objective

- Perform register, memory, and I/O transfers.

- Compare execution time.

- Analyze bottlenecks.

Theory

Single Bus

- One common bus.

- Only one transfer at a time.

- Simpler but slower.

Multi Bus

- Multiple buses.

- Parallel data transfers.

- Higher performance.
code:

single_bus = 4

multi_bus = 2

print("Single Bus")

for i in range(single_bus):

print("Cycle", i + 1)

print("\nMulti Bus")

for i in range(multi_bus):

print("Cycle", i + 1)

print("\nSingle Bus Cycles =", single_bus)

print("Multi Bus Cycles =", multi_bus)

print("Speed Improvement =", single_bus / multi_bus)

Procedure

- Design a single-bus architecture.

- Transfer:

  - Register → Register

  - Memory → Register

  - Register → I/O

- Record clock cycles.

- Repeat using a multi-bus architecture.

- Compare performance.

Example

Single Bus

| Cycle | Operation |
| --- | --- |
| 1 | Memory → Bus |
| 2 | Bus → Register |
| 3 | Register → ALU |
| 4 | ALU → Register |

Total = 4 cycles

Multi Bus

| Cycle | Operation |
| --- | --- |
| 1 | Memory → Register |
| 1 | Register → ALU |
| 2 | ALU → Register |

Total = 2 cycles

Comparison

| Parameter | Single Bus | Multi Bus |
| --- | --- | --- |
| Cost | Low | High |
| Hardware | Simple | Complex |
| Speed | Low | High |
| Parallelism | No | Yes |
| Bottlenecks | High | Low |

Result

Multi-bus organization reduced execution time by enabling simultaneous transfers.

Experiment 3: Five Stage Instruction Pipeline

Aim

To simulate a five-stage instruction pipeline and compare its performance with a non-pipelined processor.

Pipeline Stages

- Fetch (IF)

- Decode (ID)

- Execute (EX)

- Memory (MEM)

- Write Back (WB)

Sample Instructions

I1 : ADD R1,R2,R3

I2 : SUB R4,R1,R5

I3 : AND R6,R2,R7

I4 : OR R8,R1,R9

Pipeline Diagram

| Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | MEM | EX | ID | IF |
| 5 | WB | MEM | EX | ID |
| 6 |  | WB | MEM | EX |
| 7 |  |  | WB | MEM |
| 8 |  |  |  | WB |

code:

instructions = ["ADD", "SUB", "AND", "OR"]

stages = ["IF", "ID", "EX", "MEM", "WB"]

cycles = len(instructions) + len(stages) - 1

for c in range(cycles):

print("\nCycle", c + 1)

for i in range(len(instructions)):

stage = c - i

if 0 <= stage < len(stages):

print(instructions[i], "->", stages[stage])

non_pipeline = len(instructions) * 5

pipeline = cycles

print("\nNon-Pipeline Cycles =", non_pipeline)

print("Pipeline Cycles =", pipeline)

print("Speedup =", round(non_pipeline / pipeline, 2))

print("Throughput =", round(len(instructions) / pipeline, 2))

Performance

Non-Pipelined

4 instructions × 5 cycles = 20 cycles

Pipelined

5 + (4−1) = 8 cycles

Speedup

Speedup = 20 / 8 = 2.5

Throughput

Non-pipelined = 4/20 = 0.20 instructions/cycle

Pipelined = 4/8 = 0.50 instructions/cycle

Result

Instruction pipelining significantly improved throughput and reduced execution time.

Experiment 4: Pipeline Hazards

Aim

To study different pipeline hazards and methods to resolve them.

Types of Hazards

1. Data Hazard

I1 : ADD R1,R2,R3

I2 : SUB R4,R1,R5

Problem:
I2 needs R1 before I1 writes it.

Solution:

- Stall

- Data Forwarding

2. Control Hazard

BEQ R1,R2,LABEL

Problem:
Branch decision delays instruction fetch.

Solution:

- Branch Prediction

- Pipeline Flush

3. Structural Hazard

Problem:
Instruction fetch and memory access require the same memory simultaneously.

Solution:

- Separate instruction/data memory

- Multiple functional units

Comparison

| Hazard | Cause | Solution |
| --- | --- | --- |
| Data | Dependency | Forwarding, Stall |
| Control | Branch | Prediction |
| Structural | Resource Conflict | Extra Hardware |

code:

print("Data Hazard")

print("ADD R1,R2,R3")

print("SUB R4,R1,R5")

print("Resolved using Data Forwarding")

print("\nControl Hazard")

print("BEQ R1,R2,LABEL")

print("Resolved using Branch Prediction")

print("\nStructural Hazard")

print("Instruction Fetch and Memory Access")

print("Resolved using Separate Memory")

stall = 3

forward = 1

print("\nCycles with Stall =", stall)

print("Cycles with Forwarding =", forward)

Observation

| Technique | CPI |
| --- | --- |
| No Solution | 2.4 |
| Stall | 1.8 |
| Forwarding | 1.2 |
| Prediction | 1.05 |

Result

Forwarding and branch prediction minimized pipeline stalls and improved processor performance.

Experiment 5: Superscalar and Out-of-Order Execution

Aim

To study superscalar execution along with out-of-order execution, speculative execution, hyper-threading, and simultaneous multithreading (SMT).

Theory

Superscalar Processor

- Executes multiple instructions in one clock cycle.

- Uses multiple execution units.

Example:

Cycle 1

ALU1 → ADD

ALU2 → SUB

Two instructions execute simultaneously.

Out-of-Order Execution

Independent instructions execute before earlier stalled instructions.

Example

I1 Load

I2 Add

I3 Multiply

If I1 stalls, I2 and I3 execute first.

Speculative Execution

The processor predicts branch outcomes and executes instructions before the branch is resolved.

If prediction is wrong:

- Discard speculative results.

- Restart from the correct branch.

Hyper-Threading

A single physical core executes two hardware threads.

Example

Core 1

Thread A

Thread B

Improves CPU utilization.

Simultaneous Multithreading (SMT)

Multiple threads issue instructions to different execution units in the same cycle.

code:

instructions = ["LOAD", "ADD", "SUB", "MUL", "AND", "OR"]

print("Superscalar Execution")

issue_width = 2

cycle = 1

for i in range(0, len(instructions), issue_width):

print("\nCycle", cycle)

for ins in instructions[i:i + issue_width]:

print(ins)

cycle += 1

print("\nOut-of-Order Execution")

print("LOAD Waiting")

print("SUB Executed")

print("AND Executed")

print("OR Executed")

print("ADD Executed")

print("MUL Executed")

print("\nSpeculative Execution")

print("Branch Predicted Correctly")

print("\nHyper-Threading")

thread1 = ["LOAD", "ADD", "STORE"]

thread2 = ["MUL", "SUB", "OR"]

for i in range(3):

print("Thread1 ->", thread1[i])

print("Thread2 ->", thread2[i])

print("\nSMT Execution")

print("ALU1 -> Thread1 ADD")

print("ALU2 -> Thread2 SUB")

print("LOAD UNIT -> Thread3 LOAD")

scalar = 6

superscalar = 3

print("\nScalar Cycles =", scalar)

print("Superscalar Cycles =", superscalar)

print("Speedup =", scalar / superscalar)

print("Throughput =", len(instructions) / superscalar)


Example

ALU1 → Thread A

ALU2 → Thread B

Load Unit → Thread C

Performance Comparison

| Feature | Instructions/Cycle | Resource Utilization |
| --- | --- | --- |
| Scalar | 1 | Low |
| Pipeline | 1 | Medium |
| Superscalar | 2–4 | High |
| Out-of-Order | 3–5 | Higher |
| Hyper-Threading | 4–6 | Better |
| SMT | 6–8 | Maximum |

Advantages

- Higher throughput

- Better CPU utilization

- Reduced execution time

- Efficient handling of stalls

- Improved parallel execution

Result

Superscalar processors combined with out-of-order execution, speculative execution, hyper-threading, and SMT significantly increased instruction throughput and resource utilization compared with scalar and basic pipelined processors.

Code of Conduct:

I J. BRAHMAIAH(192472286) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

J. BRAHMAIAH

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |