SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL3- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | J.BRAHMAIAH | Reg.No.: | 192472286 |
| Department: | CSE-AI | Date: | 13-08-2026 |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

1. Compare L1, L2, and L3 Cache Levels in Terms of Latency, Bandwidth, and Throughput

Introduction

Cache memory is a small, high-speed memory located close to the processor. Modern CPUs generally use multiple cache levels such as L1, L2, and L3. These levels reduce the time required to access frequently used instructions and data. Each level provides a different balance between latency, capacity, bandwidth, and throughput.

Scope of Items

The comparison covers:

- L1, L2, and L3 cache

- Access latency

- Bandwidth

- Throughput

- Cache size

- Effect on processor performance

Comparison Depth

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Size | Smallest | Larger than L1 | Largest |
| Latency | Very low | Low | Higher than L1 and L2 |
| Bandwidth | Very high | High | Lower than L1 and L2 |
| Location | Closest to CPU core | Close to CPU core | Usually shared among cores |
| Access frequency | Very frequent | Frequent | Less frequent |
| Main purpose | Immediate data/instruction access | Backup for L1 | Reduce DRAM accesses |

L1 cache has the lowest latency because it is located closest to the CPU. L2 cache provides more storage but requires slightly more time to access. L3 cache is generally much larger and may be shared between processor cores, but it has higher latency.

Use of Evidence

When the processor finds required data in L1 cache, it can access it much faster than data stored in DRAM. If there is an L1 cache miss, the processor checks L2 and then L3. Only when the data is unavailable in all cache levels does it normally need to access main memory.

Therefore, a higher cache-hit rate reduces processor waiting time and improves effective memory bandwidth and system throughput.

Critical Thinking

A single large cache would provide greater capacity but would not necessarily provide the lowest latency. Smaller caches can be made faster and placed closer to CPU execution units. Therefore, multiple cache levels provide a practical balance between speed, capacity, cost, and power consumption.

Clarity of Presentation

The cache hierarchy can be represented as:

CPU → L1 → L2 → L3 → Main Memory

As the distance from the CPU increases, cache capacity generally increases while access speed decreases.

Conclusion

L1, L2, and L3 caches work together to improve processor performance. L1 provides the fastest access, L2 provides additional high-speed storage, and L3 reduces the number of accesses to slower main memory. This hierarchical arrangement increases processor throughput and reduces memory-related delays.

2. Differentiate SRAM and DRAM by Evaluating Their Latency, Bandwidth, and Throughput

Introduction

SRAM and DRAM are two important types of volatile semiconductor memory. SRAM is faster and is mainly used in processor cache, while DRAM provides higher storage density and is mainly used as system main memory. Their different characteristics determine their roles in computer memory architecture.

Scope of Items

The comparison covers:

- SRAM and DRAM

- Latency

- Bandwidth

- Throughput

- Storage density

- Refresh requirement

- Cache and main-memory applications

Comparison Depth

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Latency | Very low | Higher than SRAM |
| Bandwidth | Very high | High |
| Throughput | High for fast repeated access | High for large data transfers |
| Refresh | Not required | Required |
| Density | Lower | Higher |
| Cost per bit | Higher | Lower |
| Common use | CPU cache | Main memory |

SRAM uses flip-flop circuits to store data and does not require periodic refreshing. DRAM uses capacitors to store data and therefore requires periodic refresh operations.

Use of Evidence

SRAM's low latency makes it suitable for L1, L2, and other high-speed cache structures. DRAM is more economical and can provide much larger capacities, making it suitable for main memory.

Critical Thinking

Using SRAM for the entire main memory would provide fast access but would increase system cost and reduce achievable memory capacity. Using DRAM for processor caches would increase memory access delays. Therefore, combining SRAM and DRAM provides a better balance between performance and capacity.

Clarity of Presentation

The relationship can be represented as:

CPU → SRAM Cache → DRAM Main Memory

SRAM mainly focuses on speed, while DRAM focuses on capacity and cost efficiency.

Conclusion

SRAM provides lower latency and excellent access performance, making it ideal for cache memory. DRAM provides greater capacity at a lower cost and is therefore suitable for main memory. Both technologies are essential components of a modern hierarchical memory system.

3. Compare Virtual Memory with Paging and Cache Memory Mechanisms

Introduction

Virtual memory, paging, and cache memory are important mechanisms in computer architecture and operating systems. Cache memory primarily reduces processor memory-access latency, whereas virtual memory and paging provide efficient memory management and allow programs to use a large logical address space.

Scope of Items

The comparison covers:

- Cache memory

- Virtual memory

- Paging

- Access latency

- System throughput

- Cache hits and misses

- Page faults

Comparison Depth

| Feature | Cache Memory | Virtual Memory and Paging |
| --- | --- | --- |
| Main purpose | Reduce access latency | Manage memory and address space |
| Access latency | Extremely low | Higher |
| Primary hardware | SRAM or similar fast memory | DRAM with secondary storage support |
| Performance event | Cache hit or miss | Page hit or page fault |
| Effect on throughput | Usually increases throughput | Can decrease throughput during frequent page faults |
| Main benefit | Faster CPU data access | Efficient memory utilization and protection |

Cache stores frequently accessed data close to the CPU. Paging divides virtual memory into fixed-size pages and maps them to physical memory frames.

Use of Evidence

A cache hit allows data to be retrieved quickly. If a cache miss occurs, the processor must access a lower memory level. In paging, if a required page is not present in physical memory, a page fault occurs and the operating system may need to retrieve the page from secondary storage.

A page fault takes considerably longer than a normal cache access and can reduce system throughput.

Critical Thinking

Cache memory and virtual memory are not competing mechanisms. They solve different problems. Cache memory reduces the speed gap between the CPU and memory, while virtual memory addresses memory capacity, protection, isolation, and address-space management.

Excessive page faults can lead to thrashing, where the system spends more time transferring pages than executing useful programs.

Clarity of Presentation

The overall memory path can be represented as:

CPU → Cache → RAM → Secondary Storage

Cache improves access speed, while paging manages the movement and mapping of data between virtual memory and physical memory.

Conclusion

Cache memory primarily improves processor performance by reducing access latency. Virtual memory and paging provide efficient memory management and protection. A well-designed system uses both mechanisms together to achieve high performance and efficient memory utilization.

4. Analyze NAND Flash and DRAM with Respect to Latency, Bandwidth, and Suitability

Introduction

NAND Flash and DRAM are semiconductor memory technologies used at different levels of computer systems. DRAM is volatile and provides fast random access, while NAND Flash is non-volatile and provides persistent data storage. Their different performance characteristics determine their positions in a hierarchical memory system.

Scope of Items

The comparison covers:

- NAND Flash

- DRAM

- Access latency

- Bandwidth

- Throughput

- Persistence

- Memory hierarchy suitability

Comparison Depth

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Latency | Very low | Much higher |
| Random access | Excellent | Less efficient |
| Sequential bandwidth | High | High |
| Persistence | Volatile | Non-volatile |
| Write operation | Fast | Slower |
| Capacity | Moderate to high | Very high |
| Common application | Main memory | SSD and persistent storage |

DRAM provides rapid access to active programs and data. NAND Flash retains information without continuous power, making it suitable for long-term storage.

Use of Evidence

DRAM is significantly faster for random data access. NAND Flash can provide high sequential data-transfer rates, especially when multiple Flash chips operate in parallel. However, NAND Flash has additional program and erase operations that make its write latency much higher than DRAM.

Critical Thinking

NAND Flash cannot normally replace DRAM as the main working memory because of its higher latency and different write characteristics. However, using NAND Flash as persistent storage provides high capacity and data retention without power.

Therefore, DRAM and NAND Flash are complementary technologies rather than direct replacements.

Clarity of Presentation

A simplified hierarchy is:

CPU → Cache → DRAM → NAND Flash Storage

Each level provides a different balance of speed, capacity, and persistence.

Conclusion

DRAM is better suited for active working memory because of its low latency and strong random-access performance. NAND Flash is better suited for persistent storage because of its non-volatility and high storage density. Combining both technologies creates an efficient memory and storage hierarchy.

5. Compare MRAM and RRAM in Terms of Access Time, Throughput, and Energy Efficiency

Introduction

MRAM and RRAM are emerging non-volatile memory technologies designed to overcome some limitations of traditional memory technologies. MRAM stores information using magnetic states, whereas RRAM stores information by changing the resistance of a memory element. Both technologies offer opportunities for faster and more energy-efficient memory systems.

Scope of Items

The comparison covers:

- MRAM and RRAM

- Access time

- Throughput

- Energy efficiency

- Endurance

- Non-volatility

- Suitability for future memory systems

Comparison Depth

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Access time | Very fast | Fast, depending on implementation |
| Read performance | High | High |
| Write performance | High | Can vary |
| Throughput | High | Potentially high |
| Energy efficiency | Very good | Potentially very good |
| Non-volatility | Yes | Yes |
| Endurance | Generally high | Depends on technology |
| Density | Good | Potentially very high |

MRAM uses magnetic states to represent information. Because it is non-volatile, it can retain information when power is removed. RRAM uses changes in electrical resistance to store data and has potential for highly dense memory structures.

Use of Evidence

MRAM is known for fast access, high endurance, and low standby power. RRAM can offer high density and low-power operation, although its practical performance depends on device design and manufacturing technology.

Critical Thinking

MRAM provides a strong combination of speed, endurance, and non-volatility, making it attractive for applications requiring frequent read and write operations. RRAM offers potential advantages in density, scalability, and energy efficiency.

However, both technologies face challenges involving manufacturing complexity, reliability, cost, and large-scale commercialization. Therefore, their future role depends on improvements in fabrication and device reliability.

Clarity of Presentation

The comparison can be summarized as:

MRAM → Fast access + High endurance + Non-volatility

RRAM → High density + Low power potential + Non-volatility

Conclusion

MRAM and RRAM are promising technologies for future memory architectures. MRAM is particularly attractive when fast access and high endurance are required, while RRAM offers strong potential for high-density and energy-efficient memory. Both technologies could complement or replace certain conventional memory technologies as their manufacturing and reliability continue to improve.

Code of Conduct:

I J. BRAHMAIAH (192472286) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

J. BRAHMAIAH

Signature of the student:

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |