# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

### COURSE NAME:	COMPUTER ARCHITECTURE	
COURSE CODE:	CSA1201

### COURSE OUTCOME COVERED

CO1: Components of a computer system, including CPU, memory, and I/O. Basic Operational Concepts: Instruction execution cycle, instruction fetch and decode. Bus Structures: Single-bus, multi-bus, and hierarchical buses. Factors of Performance: CPU execution time, CPI, clock cycles, and benchmarking.8085 and 8086 Architectures: Overview, instruction sets, and addressing modes. Modern Architectures: ARM Cortex, Intel Core, and AMD Ryzen. Instruction Sets, Formats, and Addressing Modes. Number Systems: Binary, hexadecimal, and their applications in computer systems. (BL4)

Assessment Tool Weightage: 40%

### QUESTIONS:5	Total Marks:25

ANNEXURE A APPLICATION BASED QUESTIONS

Code of Conduct:

I    J. BRAHMAIAH [192472286] certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

J. BRAHMAIAH

Signature of the student:

### ANNEXURE A

- A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

1. Understanding of Problem Context

A smart city surveillance system continuously collects video feeds from multiple cameras installed in public places such as roads, railway stations, airports, and shopping areas. These video streams are transferred through Input/Output (I/O) devices to the computer's memory (RAM), where the CPU processes the data to detect suspicious activities in real time. During peak hours, hundreds of cameras send high-resolution video simultaneously, creating a heavy workload. As a result, the CPU, memory, and I/O devices become overloaded, causing lag and delayed threat detection. This affects public safety because important events may not be detected immediately.

2. Application of Concepts

Interaction of CPU, Memory, and I/O

CPU (Central Processing Unit)

- Executes image processing and AI algorithms.

- Fetches instructions and video data from memory.

- Processes the data and generates the output.

Memory (RAM)

- Temporarily stores video frames and program instructions.

- Supplies data to the CPU for processing.

- Stores processed results before sending them to storage or display.

I/O Devices

- Cameras continuously capture video and send it to the system.

- Storage devices and displays receive processed information.

- Large amounts of I/O traffic increase system load.

Bus Structures

Single-Bus Architecture

- CPU, memory, and I/O devices share one communication bus.

- Only one data transfer occurs at a time.

- Heavy traffic creates bus congestion and increases CPU waiting time.

Advantages

- Simple design.

- Low implementation cost.

Disadvantages

- Bus bottleneck.

- Higher latency.

- Poor performance during heavy workloads.

Multi-Bus Architecture

- Separate buses are provided for CPU, memory, and I/O communication.

- Multiple transfers occur simultaneously.

- Reduces congestion and improves throughput.

Advantages

- Faster communication.

- Better processor utilization.

- Suitable for real-time surveillance systems.

Disadvantages

- More complex architecture.

- Higher implementation cost.

Instruction Execution Cycle

The CPU processes every instruction through four stages:

- Fetch – Retrieve instructions from memory.

- Decode – Interpret the instruction.

- Execute – Perform image processing or AI operations.

- Store (Write Back) – Save the processed result.

Performance can be improved using:

- Instruction Pipelining – Executes multiple instructions simultaneously.

- Cache Memory – Reduces memory access time.

- Direct Memory Access (DMA) – Transfers data between I/O devices and memory without continuous CPU involvement.

- Multi-Core Processors – Process multiple video streams in parallel.

- Faster Memory and Wider Buses – Increase data transfer speed.

3. Problem-Solving Approach

To reduce lag and improve threat detection, the surveillance system should:

- Replace the single-bus architecture with a multi-bus architecture.

- Increase cache memory to reduce RAM access.

- Use DMA for faster data transfer.

- Implement instruction pipelining to execute multiple instructions simultaneously.

- Deploy multi-core processors for parallel video processing.

- Upgrade RAM bandwidth to support continuous high-speed video transfers.

These improvements reduce CPU waiting time, increase processing speed, and ensure smooth communication between hardware components.

4. Accuracy of Solution

The delay occurs because continuous video streams overload the communication between CPU, memory, and I/O devices. A single-bus architecture creates a bottleneck since all components compete for the same communication path. A multi-bus architecture allows simultaneous data transfers, significantly reducing waiting time. Techniques such as cache memory, DMA, instruction pipelining, and multi-core processors improve CPU execution efficiency, reduce clock cycles, and increase overall system performance. Benchmarking can also be used to measure CPU execution time, CPI, memory throughput, and system response, helping engineers identify and remove performance bottlenecks.

5. Conclusion

The performance of a smart city surveillance system depends on efficient communication between the CPU, memory, and I/O devices. During peak hours, heavy video traffic causes delays due to bus congestion and increased CPU waiting time. Replacing a single-bus architecture with a multi-bus architecture, along with instruction pipelining, cache memory, DMA, multi-core processors, and benchmarking, significantly improves processing speed, reduces response time, and enables accurate real-time threat detection. This results in a faster, more reliable, and highly efficient surveillance system that meets the requirements of modern smart cities.

- A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch–decode–execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

1. Understanding of Problem Context

A real-time stock trading application processes millions of buy and sell transactions every second. The processor must execute instructions quickly to ensure that users receive instant updates and successful transactions. During periods of high trading activity, users experience delays because the processor has a high Cycles Per Instruction (CPI). Frequent memory access and branch instructions increase the number of clock cycles required to execute each instruction. As a result, transaction processing becomes slower, reducing system performance and affecting the reliability of financial operations.

2. Application of Concepts

Fetch–Decode–Execute Cycle

Every instruction is processed through the following stages:

1. Fetch

- The CPU fetches instructions and required data from memory.

- If data is not available in cache memory, the CPU waits for slower RAM, increasing execution time.

2. Decode

- The Control Unit interprets the fetched instruction.

- Branch instructions require the processor to determine the next instruction, causing delays if prediction is incorrect.

3. Execute

- The Arithmetic Logic Unit (ALU) performs arithmetic and logical operations.

- Memory-related instructions require additional access to RAM, increasing CPI.

4. Write Back (Store)

- The processed result is stored in registers or memory.

- Frequent memory writes increase execution time.

Relationship Between CPI and CPU Performance

CPU execution time depends on:

- Instruction Count (IC)

- Cycles Per Instruction (CPI)

- Clock Cycle Time

A higher CPI means the CPU requires more clock cycles for each instruction, increasing total execution time and reducing overall performance.

3. Problem-Solving Approach

The following architectural enhancements can reduce CPI and improve processor performance:

Instruction Pipelining

- Executes multiple instructions simultaneously in different stages.

- Improves instruction throughput.

Cache Memory

- Stores frequently used instructions and data close to the CPU.

- Reduces memory access delays.

Branch Prediction

- Predicts the outcome of branch instructions.

- Minimizes pipeline stalls and incorrect instruction fetching.

Out-of-Order Execution

- Executes independent instructions while waiting for memory operations.

- Improves CPU utilization.

Superscalar Architecture

- Executes multiple instructions in a single clock cycle.

- Increases parallel processing capability.

Multi-Core Processors

- Distributes multiple trading requests across different CPU cores.

- Handles heavy workloads efficiently.

Faster Memory System

- Uses larger cache, faster RAM, and prefetching techniques.

- Reduces memory latency.

4. Accuracy of Solution

The delays occur because frequent memory accesses and branch instructions increase the processor's CPI during the fetch–decode–execute cycle. Architectural enhancements such as instruction pipelining, cache memory, branch prediction, out-of-order execution, superscalar processors, and multi-core CPUs significantly reduce execution time. Benchmarking tools can measure CPU execution time, CPI, throughput, and memory performance, allowing engineers to identify bottlenecks and optimize system performance. These improvements ensure faster transaction processing even during periods of heavy trading.

5. Conclusion

A real-time stock trading application requires high-speed instruction execution to process financial transactions without delay. High CPI caused by frequent memory access and branch instructions reduces CPU performance and increases execution time. By implementing instruction pipelining, cache memory, branch prediction, out-of-order execution, superscalar architecture, faster memory systems, multi-core processors, and benchmarking techniques, the processor can reduce CPI, improve throughput, and deliver faster and more reliable transaction processing under high workloads. These enhancements ensure efficient CPU utilization and maintain the performance required for modern real-time trading systems.

- An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

1. Understanding of Problem Context

An industrial temperature control system uses an 8085 microprocessor to monitor temperature through sensors and control devices such as heaters and cooling fans. The sensors continuously provide temperature data, which is processed by the microprocessor before sending control signals to the actuators. However, the system occasionally produces incorrect temperature readings because of the improper use of addressing modes in the program. When the processor accesses the wrong memory location or register, it processes invalid data, resulting in incorrect temperature control. Such errors may cause overheating, equipment damage, increased energy consumption, and reduced industrial safety. Therefore, selecting the correct addressing mode and using the 8085 instruction set properly are essential for reliable system operation.

2. Application of Concepts

Addressing Modes in 8085

The 8085 microprocessor supports different addressing modes that determine how data is accessed.

Immediate Addressing

- Data is included directly in the instruction.

- Example: MVI A,25H

- Used for assigning fixed values.

Register Addressing

- Data is stored in CPU registers.

- Example: MOV A,B

- Provides fast data access.

Direct Addressing

- Data is accessed from a specific memory location.

- Example: LDA 2500H

- Suitable for reading sensor values stored in memory.

Register Indirect Addressing

- Memory address is stored in the HL register pair.

- Example: MOV A,M

- Useful for processing multiple sensor readings.

Implied Addressing

- Operand is automatically specified by the instruction.

- Example: CMA

- No explicit operand is required.

Possible Causes of Errors

- Incorrect memory address selection.

- Improper initialization of the HL register pair.

- Register values being overwritten.

- Incorrect use of data transfer instructions.

- Invalid or corrupted sensor data stored in memory.

3. Problem-Solving Approach

To improve the accuracy and reliability of the temperature control system, the following measures should be implemented:

- Use Direct Addressing for fixed sensor memory locations.

- Initialize registers correctly before data access.

- Validate sensor readings before processing.

- Use appropriate instructions such as MOV, LDA, STA, IN, and OUT for correct data transfer.

- Compare temperature values using CMP and apply conditional instructions like JZ, JC, and JMP for decision-making.

- Regularly test and debug the program to detect addressing errors.

- Calibrate sensors periodically to ensure accurate input data.

These measures ensure correct data access and improve the overall reliability of the embedded system.

4. Accuracy of Solution

The 8085 instruction set provides efficient instructions for data transfer, arithmetic operations, logical operations, branching, and I/O communication.

- Data Transfer: MOV, LDA, STA

- Arithmetic: ADD, SUB

- Logical: CMP, ANA, ORA

- Branching: JMP, JZ, JC

- Input/Output: IN, OUT

Using the correct addressing mode with these instructions ensures that the processor reads valid sensor data, processes it accurately, and controls the actuators correctly. Proper register initialization, memory management, and program testing reduce processing errors and improve system stability.

5. Conclusion

The performance of an 8085-based industrial temperature control system depends on the correct use of addressing modes and instruction sets. Improper memory access can produce incorrect temperature readings and unsafe equipment operation. Selecting suitable addressing modes, properly initializing registers, using appropriate 8085 instructions, and validating sensor inputs significantly improve system accuracy and reliability. These practices ensure efficient temperature monitoring, safe actuator control, and dependable industrial automation.

- A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

1. Understanding of Problem Context

A computing system uses an 8086 microprocessor to execute multiple programs simultaneously. The processor organizes memory using segmentation, which divides memory into logical sections such as the Code Segment (CS), Data Segment (DS), and Stack Segment (SS). These segments help the processor store instructions, data, and temporary information separately. However, if segment registers are not managed correctly, the system may access incorrect memory locations or experience stack overflow errors. These problems can lead to incorrect program execution, data corruption, system crashes, and poor multitasking performance. Therefore, proper memory segmentation, instruction execution, and addressing mode management are essential for reliable operation.

2. Application of Concepts

Segmentation in 8086

The 8086 microprocessor supports 1 MB of memory using 16-bit segment registers and offset addresses.

Code Segment (CS)

- Stores program instructions.

- Works with the Instruction Pointer (IP).

- The CPU fetches instructions using the CS:IP address.

Data Segment (DS)

- Stores variables, constants, and program data.

- Used by most data transfer instructions.

Stack Segment (SS)

- Stores temporary data, return addresses, and register values.

- Works with the Stack Pointer (SP).

- Used by PUSH, POP, CALL, and RET instructions.

Problems Due to Improper Segment Handling

- Incorrect CS causes the CPU to execute wrong instructions.

- Incorrect DS leads to invalid data access.

- Incorrect SS or SP results in stack overflow or stack underflow.

- Improper segment switching during multitasking may allow one program to access another program's memory.

Addressing Modes Used

- Immediate Addressing

- Register Addressing

- Direct Addressing

- Register Indirect Addressing

- Based and Indexed Addressing

These addressing modes help the processor access memory efficiently.

3. Problem-Solving Approach

The following measures improve program execution and prevent memory errors:

- Initialize CS, DS, and SS before program execution.

- Maintain the correct Stack Pointer (SP) during stack operations.

- Use PUSH and POP instructions carefully.

- Select the appropriate addressing mode based on the application.

- Validate memory addresses before accessing data.

- Save and restore segment registers during program switching.

- Perform regular debugging and testing to identify segmentation errors.

These solutions improve memory management and ensure reliable multitasking.

4. Accuracy of Solution

The 8086 executes instructions through the Fetch–Decode–Execute–Store cycle.

- Fetch: Retrieves instructions from the Code Segment.

- Decode: The Control Unit interprets the instruction.

- Execute: The CPU performs arithmetic, logical, or memory operations.

- Store: The result is written back to memory or registers.

Proper initialization of CS, DS, SS, IP, and SP ensures that instructions, data, and stack operations access the correct memory locations. Using appropriate addressing modes further improves execution accuracy and system stability. Benchmarking and program testing can also identify segmentation and memory-related issues before deployment.

5. Conclusion

The segmented memory architecture of the 8086 microprocessor improves memory organization by separating program instructions, data, and stack into different segments. However, incorrect management of segment registers can cause invalid memory access, stack overflow, and program execution errors. Proper initialization of segment registers, careful stack management, correct addressing mode selection, and regular program testing ensure accurate instruction execution and reliable multitasking performance. These practices make the computing system more efficient, stable, and suitable for handling multiple programs simultaneously.

- A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

1. Understanding of Problem Context

A data communication system represents packet information in hexadecimal (Hex) format because it is compact, easy to read, and simplifies debugging. However, the CPU executes all instructions in binary format. Therefore, hexadecimal values must be accurately converted into binary before processing. In the given scenario, a software engineer incorrectly interprets hexadecimal values, resulting in wrong binary data. This causes incorrect instruction execution, packet processing errors, and communication failures. In real-time applications such as banking, healthcare, and industrial automation, such mistakes can reduce system reliability and affect overall performance. Therefore, accurate number system conversion and efficient CPU design are essential for reliable data communication.

2. Application of Concepts

Importance of Hexadecimal to Binary Conversion

Hexadecimal is widely used because it provides a shorter representation of binary numbers.

- One hexadecimal digit represents 4 binary bits.

- Example:

  - Hexadecimal: 3A

  - Binary: 0011 1010

- Memory addresses, machine instructions, and network packets are commonly represented in hexadecimal.

- Accurate conversion ensures the CPU correctly interprets instructions and processes packet data.

Effects of Conversion Errors

Incorrect conversion can cause several problems:

- Wrong binary instructions are generated.

- The CPU may execute incorrect instructions.

- Invalid memory addresses may be accessed.

- Packet headers and payloads may be interpreted incorrectly.

- Data corruption and communication failures may occur.

- Real-time systems may experience delayed responses or system crashes.

Role of Efficient CPU Design

Modern processors improve reliability by using:

- Instruction pipelining for faster instruction execution.

- Cache memory to reduce memory access time.

- Error detection and exception handling to identify invalid instructions.

- High-speed buses for faster communication between CPU and memory.

- Efficient instruction decoding for accurate binary interpretation.

3. Problem-Solving Approach

The following methods help minimize conversion errors and improve system performance:

- Use verified hexadecimal-to-binary conversion algorithms.

- Validate packet data before processing.

- Implement parity bits, checksums, or CRC for error detection.

- Use efficient CPU architectures with cache memory and pipelining.

- Perform regular software testing to identify conversion errors.

- Benchmark the system under different workloads to detect performance bottlenecks.

These measures improve data accuracy and ensure reliable communication.

4. Accuracy of Solution

Benchmarking is an important performance evaluation technique that measures:

- CPU execution time.

- Clock cycles.

- Cycles Per Instruction (CPI).

- Throughput.

- Memory performance.

These metrics help engineers identify conversion errors, memory bottlenecks, and processor inefficiencies. Efficient CPU architecture combined with benchmarking ensures accurate instruction execution, faster packet processing, and reliable real-time communication. Correct hexadecimal-to-binary conversion also prevents data corruption and improves system stability.

5. Conclusion

Hexadecimal and binary number systems play an important role in modern data communication systems. While hexadecimal simplifies data representation, the CPU processes instructions only in binary format. Incorrect conversion can lead to wrong instruction execution, communication failures, and reduced system performance. By using accurate conversion techniques, efficient CPU architecture, error detection methods, and benchmarking, organizations can improve processing speed, maintain data integrity, and ensure reliable real-time communication. These practices make the system more efficient, secure, and dependable for critical applications.

### MARKS ALLOCATION

| Criteria | Understanding of Problem Context (20%) | Application of Concepts (20%) | Problem-Solving Approach (20%) | Accuracy of Solution (20%) | Clarity (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

| Criteria | Weightage | Leel 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understan ding of Problem Context | 20 | Clearly understands the problem and accurately identifies all requirements | Good understanding with minor gaps | Basic understanding; some requirements unclear | Poor understanding or misinterpretation of the problem |
| Application of Concepts | 30 | Accurately applies appropriate concepts to solve complex problems | Applies relevant concepts correctly with minor errors | Applies basic concepts with limited accuracy | Fails to apply appropriate concepts |
| Problem-Solving Approach | 20 | Logical, well-structured, and efficient approach | Mostly logical approach with minor gaps | Basic approach with some inconsistencies | Illogical or unclear approach |
| Accuracy of Solution | 20 | Completely correct solution with proper justification | Mostly correct with minor errors | Partially correct solution | Incorrect or incomplete solution |
| Clarity | 10 | Clear, well-organized, and properly explained solution | Understandable with minor clarity issues | Limited clarity and explanation | Poorly presented and difficult to understand |

### RUBRICS FOR CALCULATION: