SIMATS ENGINEERING

Department of Computer Science & Engineering

| Course Code: | CSA1205 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL 2- DEBUGGING AND OPTIMISATION TASK |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | J. BRAHMAIAH | Reg.No.: | 192472286 |
| Department: | CSE_ AI | Date: | 28-07-2026 |

ASSESSMENT TOOL 2- ANALYTICAL PROBLEM SOLVING

These are debugging and optimization questions, so the answers should explain the error, its cause, the correction, and the optimized solution. Below are concise but sufficiently detailed exam-ready answers.

Q1. Booth's Multiplication Debugging (-13 × 9)

Problem Understanding

A processor implementing Booth's multiplication produces an incorrect result for −13 × 9. The task is to identify the incorrect addition/subtraction or shift operation, correct it, and optimize the algorithm to reduce execution cycles.

Method Selection

The Booth's multiplication algorithm uses:

- Multiplicand (M)

- Multiplier (Q)

- Accumulator (A)

- Booth bit (Q₋₁)

Each iteration examines Q₀ and Q₋₁.

| Q₀Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

An Arithmetic Right Shift (ARS) is performed after each operation.

Solution Process

Binary Representation

-13 = 11110011 (8-bit)

+9 = 00001001

Suppose during the third iteration,

Instead of

Q0Q-1 = 10

A = A − M

the processor performs

A = A + M

This changes the accumulator value and all subsequent shifts produce an incorrect product.

Another common error is performing a logical right shift instead of an arithmetic right shift, causing the sign bit to be lost.

Correct Algorithm

If Q0Q-1 = 01

A = A + M

If Q0Q-1 = 10

A = A − M

Otherwise

No operation

↓

Arithmetic Right Shift

Optimization

- Skip addition/subtraction for 00 and 11.

- Combine consecutive 1's into a single add/subtract operation.

- Use arithmetic shifts only.

- Minimize unnecessary register updates.

Accuracy of Calculation

Expected result

−13 × 9 = −117

After correcting the faulty subtraction and using arithmetic shifts, the processor produces the correct product.

Interpretation of Results

The error occurred because the wrong arithmetic operation or incorrect shift was performed during one iteration. Correcting the operation and using arithmetic right shifts preserves the sign and produces the correct signed multiplication result while reducing execution cycles.

Q2. Restoring Division Debugging (29 ÷ 5)

Problem Understanding

A processor implementing restoring division generates an incorrect quotient while dividing 29 by 5.

Method Selection

Restoring Division performs:

- Shift Left

- Subtract Divisor

- If remainder becomes negative

  - Restore remainder

  - Set quotient bit to 0

- Otherwise

  - Keep remainder

  - Set quotient bit to 1

Solution Process

Binary representation

29 = 11101

5 = 00101

Suppose after subtraction

A = 11110

The processor incorrectly sets

Q = 1

without restoring the remainder.

Correct operation

11110

+00101

------

00011

The quotient bit should be

0

The processor then proceeds to the next iteration.

Optimization

- Restore only when remainder is negative.

- Use a dedicated restore control signal.

- Reduce unnecessary memory accesses.

Accuracy of Calculation

Correct result

29 ÷ 5

Quotient = 5

Remainder = 4

Verification

5 × 5 + 4 = 29

Interpretation of Results

The incorrect quotient was caused by failure to restore the negative remainder. Restoring the accumulator before the next iteration ensures correct quotient generation and accurate division.

Q3. Non-Restoring Division Debugging

Problem Understanding

After the fourth iteration, the quotient bits become incorrect.

Method Selection

Non-restoring division avoids restoring after every negative remainder.

Rules:

Positive remainder

Subtract divisor

Negative remainder

Add divisor

Solution Process

The error occurs because the processor continues subtracting even when the remainder is negative.

Correct logic

If A ≥ 0

A = A − M

Else

A = A + M

Another mistake

Quotient bit = 1

when remainder is negative.

Correct rule

Positive remainder → Quotient = 1

Negative remainder → Quotient = 0

If the final remainder is negative,

A = A + M

must be performed.

Accuracy of Calculation

After correction,

Quotient = Correct

Remainder = Correct

Verification

Dividend = Divisor × Quotient + Remainder

Interpretation of Results

Correct quotient generation depends on checking the sign of the remainder after every iteration. Proper correction eliminates the logic error and produces accurate results.

4. Booth Multiplier Using Logical Shift

Problem Understanding

A Booth multiplier performs logical right shifts instead of arithmetic right shifts, producing incorrect signed multiplication.

Method Selection

Signed multiplication requires Arithmetic Right Shift (ARS).

Logical Right Shift inserts

0

Arithmetic Right Shift copies

MSB

Solution Process

Example

Negative number

11110010

Logical Shift

01111001

Sign changes

Incorrect

Arithmetic Shift

11111001

Sign preserved

Correct

Therefore Booth's algorithm must always use arithmetic shifts.

Optimization

- Replace logical shifter with arithmetic shifter.

- Preserve MSB during every shift.

- Implement sign-extension hardware.

Accuracy of Calculation Arithmetic shifts maintain the correct sign.

Signed multiplication produces the correct product.

Interpretation of Results

Logical shifts destroy the sign of negative numbers. Arithmetic shifts preserve the sign bit, making Booth's algorithm suitable for signed multiplication.

5. IEEE 754 Floating-Point Addition Debugging

Problem Understanding

A floating-point addition program produces incorrect results because the exponents are not aligned.

Method Selection

IEEE 754 addition requires:

- Compare exponents.

- Shift smaller mantissa.

- Add mantissas.

- Normalize.

- Round.

Solution Proces

Example; 13.25=1.10101 × 2³6.5=1.101 × 2²

Incorrect program

Adds mantissas directly

Without exponent alignment

Wrong

1.10101

+

1.101

Correct

Shift the smaller exponent.

1.101 × 2²

↓

0.1101 × 2³

Now

1.10101+0.11010

Normalize

Round

Store

Optimization

- Compare exponents before addition.

- Shift only the smaller mantissa.

- Use guard, round, and sticky bits.

- Normalize only once after addition.

Accuracy of Calculation

After alignment,

The processor produces the correct IEEE 754 result with proper normalization and rounding.

Interpretation of Results

Exponent alignment is the most important step in floating-point addition. Without it, mantissas represent different scales, producing incorrect results. Proper alignment, normalization, and rounding ensure accurate floating-point computations while minimizing precision loss.

Code of Conduct:

I J. BRAHMAIAH (192472286) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

J. BRAHMAIAH

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |